This information is old, from 2022. Please read the FINAL_RELEASE.txt file for context…

A little bit of background...

I bought an Acurite Atlas with HD Display about 5 years ago, and everything worked great. In September of 2020 a thunderstorm came through one morning and there was a power surge, afterwards the HD Display displayed the boot screen, and never changed. To me this indicated the HD Display had a corrupted application software load. I contacted Acurite about getting it reprogrammed and their response was – “Go buy another HD Display.” After regaining my composure, I realized I had no other choice but to go buy another HD Display for ~$100.

I kept the old HD Display and put it on my “to do list” to disassemble it and see what I could learn from it. What I eventually discovered in early 2022 was a main MIPS CPU board and an RF receiver module connected to it with a 6-wire ribbon cable (see picture “Inside of Acurite HD Display.jpg”). I realized I couldn’t do anything with the CPU board, but the RF receiver module was an entirely different matter. The 6-wire connections were conveniently labeled: GND, VCC, in_clk, in_data, out_data, out_clk. I immediately realized I could “bit-bang” this synchronous interface on a Raspberry Pi GPIO, and so I began this project.

Side Note: The 4-wire ribbon cable on the left side of the RF module connects to a small circuit board that contains the indoor temperature and humidity sensor. This little circuit board was actually manufactured attached to the RF module, but during production is detached and relocated so that the heat produced by the HD Display doesn’t affect the indoor temperature reading, much. As you can see in one of the pictures, I reattached this small circuit board to the RF module, but I never use the data from it in weather.c.

I was determined to use the scavenged RF receiver module to come up with a display that was larger, easier to read, and had much better graphs of the weather data.

The first step was to determine the interface protocol between the CPU and RF receiver. I asked Acurite if they would provide that info and they refused, even after I offered to sign an NDA to get it. So I attached my logic analyzer to the interface on my good HD Display; Yes, I know that voided the warranty (see pictures “Logic analyzer attached to good Acurite HD Display.jpg” and “Logic analyzer sampling data from Acurite HD Display.jpg”). What I discovered was a “unique” synchronous interface implementing 27-byte data packet reads. The protocol is a 64-bit address field (200uS clock period) sent to the RF receiver module, with the packet type (8-bits) repeated in the first and last 8-bits, and all others bits zero. The RF receiver module responds with a 27-byte packet (600uS clock period), with the first byte being the packet type, 24-bytes of data, 1-byte packet counter (why?), and a simple 8-bit checksum. Interestingly, during the 27-byte transfer from the RF receiver module, the data line from the CPU has its state toggled for each bit (why?). I’ve scanned all 256 possible packet types, and the only valid packet types the RF receiver module responds to are: 0x01, 0x02, 0x03, 0x10, and 0x44.

Packet type 0x01 contains:

- Inside temperature in both Celsius and Fahrenheit in BCD format

- Inside humidity percent in unsigned character format

Packet type 0x02 contains:

- Atmospheric pressure in both hPa and inHg in BCD format for both a reported value and what appears to be a secondary value (why?)

Packet type 0x03 contains:

- Channel number

- Battery low flag

- Outside temperature in both Celsius and Fahrenheit in BCD format

- Outside humidity percent in unsigned character format

- Lightning strike count in unsigned character format

- Lightning distance in both kilometers and miles in BCD format

Packet type 0x10 contains:

- UV index in unsigned character format

- Light (lux / 10) in unsigned 16-bit integer format

- Wind speed in both kph and mph in unsigned character format

- Wind direction degrees in BCD format

- Rain in both mm and inches in BCD format

The 0x44 packet type only ever contains 0x44 in bytes 1 and 8, so I think it’s a programming bug in the RF receiver module.

The BCD data threw me as it wasn’t consistent with what I’ve seen from several other Raspberry Pi projects using generic 433MHz RF receiver modules. Clearly Acurite’s RF receiver module is doing some data conversion before passing the data to the CPU board. I don’t know what Acurite’s Access unit uses for its RF receiver module, but I suspect it’s the same one.

More detail on these packets can be found in an Excel spreadsheet (Data Analysis.xlsx) I made during the decoding period.

The second step was connecting the RF receiver module to the Raspberry Pi’s GPIO and bit-bang the interface. I soldered a 2mm 6-pin header onto the RF receiver module in place of the 6-wire flat cable, and made a 6” cable (100mil connector for RPi GPIO and 2mm connector for RF receiver module) to make it easier to connect it all together. This also made it easy for the finishing the project as I did development work on a Raspberry Pi 3B+ but finished on a Raspberry Pi Zero 2W.

Not wanting to reinvent the wheel, I used WiringPi (ht Gordon) as my method for “bit-banging” the Raspberry Pi GPIO interface. I wrote an ARM assembly program (rf_mod_sync_io.s) that would initialize the GPIO interface and read a packet from the RF receiver module. In the initialization call (RF_Mod_Init), the C program passes the address of a 27-character array in r0. In the read call (RF_Rd_Pkt), the specific packet type to be read is passed in the lower byte of r0, the ARM assembly program reads the packet and returns the data in the 27-charater array specified in the initialization call. The ARM assembly program abides by all the “unique” Acurite timing/behavior observed on the logic analyzer. The ARM assembly program reports an error for the following conditions: 1) the RF receiver module is taking too long (i.e. time-out), 2) the checksum fails (byte 26 = 8-bit sum of bytes 0-25), or 3) the packet is empty (i.e. bytes 1-24 are all zeros).

NOTE: I discovered the system usleep function is a bit “inaccurate” for very small delays (~100uS). After much tweaking and verifying on the logic analyzer I found calling usleep with a value of 25 gives a ~100uS delay.

Actually getting the weather station data from the RF receiver module was the easy part and I was done in late June. I spent all of July and August just tinkering with weather.c on how I was going to use the data. I entertained several different appearances on my display (1920x1080), made some personal decisions on how I liked the look, and finally decided to stop changing it in late August.

I’m including everything I created during this project in this release (test/weather C programs, RF receiver module ARM assembly program, fonts, tidbits in text files, spreadsheets, etc.).

Other items needed for this project are:

WiringPi

source:	 https://github.com/WiringPi/WiringPi

Simple ARM Graphics Library

source: https://github.com/ScienceABC123/Simple-ARM-Graphics-Library

And of course, an Acurite RF receiver module. I’m sure there must be a few Acurite HD displays with corrupted software loads floating around, but may be hard to find. As I said, I began this project only after my Acurite HD Display got corrupted.

I’m making everything public domain effective immediately.

Enjoy!

weather.c read me...

The weather.c program brings everything together: initialization, sampling weather data from the RF receiver module, averaging the data over one minute intervals, computing things like dew point and displaying/graphing the data. I played around with various formats and visual arraignments, and finally decided on a six panel approach. Early on I decided on using a 24-inch 1920x1080 monitor and that I was going to hang this on the living room wall in place of my Acurite HD Display, so I also wanted it to include the time. That eventually grew into day, date and time. See picture “Wind Gust!.jpg” for my final project displayed on my living room wall.

The code begins with a few functions (sources included) to:

- Draw a graph background

- Calculate the dew point

- Calculate the wind chill

- Calculate the heat index

- Calculate solar radiation from lux

- Compute the y offset for graphing

The main program includes notes for setting the Raspberry Pi to autostart weather.c on boot up.

First, the screen is cleared to prevent the GPU from painting over the intended display.

Next, the graphics screen is initialized by opening the frame buffer and mapping it to user memory for program use. If the frame buffer isn’t 32bpp, necessary for Simple ARM Graphics Library, main will display a failure message and terminate.

Next, WiringPi is initialized by calling the initialization routine (RF_Mod_Init) in the ARM assembly program (rf_mod_sync_io.s). This is also where the packet buffer address is passed to the ARM assembly program. If WiringPi fails to initialize, main will display a failure message and terminate.

Note: All WiringPi calls, including initialization, are made from within the ARM assembly program.

Next, main sleeps for 1 second just to give you a chance to read the initialize messages.

Next, main clears the screen (again) and turns off the system cursor. The Simple ARM Graphics Library is initialized, some graphing constants are calculated, the graphics screen is cleared to black, a patience message is display in the lower left-hand corner, some one time initialized variables are initialized, and the code falls into the main loop.

Main Loop:

First, the main loop begins with sampling the weather data six times, 10 seconds apart. Wind speed is sampled every 10 seconds. Temperature, relative humidity, UV index, lux, and wind direction are sampled every 30 seconds. The battery low flag, lightning strikes, lightning distance, and precipitation are sampled in the last sampling. The weather data is averaged over the one minute sampling period.

Next, the main loop gets the current date/time of the system. It checks if the current time is the same as the last time (this always fails on the first iteration), if it is the same time it loops here until the next minute arrives. Otherwise it updates the time displayed in the lower right-hand corner of the display.

Next, the main loop checks if the day is the same as the last day time (this always fails on the first iteration), if it isn’t it updates the day/date in the lower left-hand corner of the display, resets the variables for a new day, and flags all the graphs for updating.

Next, the main loop computes the minute of the day for saving weather information of the current day, and the corresponding x-position for graphing purposes.

Next, the main loop checks the battery low flag and if set displays a battery low graphic (a vertical single cell battery symbol with a single red rectangle near its bottom) between the day/date and time at the bottom of the display (this is a big improvement on the very small icon used by the Acurite HD Display).

The main loop then proceeds to process each of the six panel areas (temperature, wind speed, relative humidity, wind direction, precipitation, and solar radiation) separately before looping back to the beginning of the main loop.

What is displayed in each panel area is position relative to the area’s x,y graph location. I did this early on as I was moving things around on the display and this made it easy to do so. Each panel’s x,y location is calculated at the beginning of its code section.

Temperature Area:

The values for current temperature, high/low, and wind chill/heat index (if applicable) are displayed.

Note: Since wind chill only exists at/below 50°F and heat index only exists at/above 75°F they are mutually exclusive and are displayed at the same location.

The temperature, dew point and wind chill/heat index (if applicable) are graphed. The colors are matched to the text labels. The graph auto-scales up/down in 10°F increments depending on the maximum and minimum values.

Note: I originally graphed the temperature before the dew point, but when I recently had 100% humidity the temperature line became hidden behind the dew point line. So I reversed the graphing order so the temperature line is more prominent.

A description (arbitrary) of the current temperature is also displayed.

Wind Speed Area:

The values for the current wind speed (averaged over the last five minutes) and the peak wind speed are displayed.

The wind speed and any wind gusts (if applicable) are graphed. The colors are matched to the text labels. The graph scale defaults to 0-20mph, but does auto-scale up in 5mph increments as necessary.

Note: Gusts are defined by NOAA as wind peak >= 18mph, AND wind peak - wind lull >= 10mph.

When a wind gust is detected, the message "Wind Gust!" will be displayed just below the wind peak.

A description (Beaufort Scale) of the current wind speed is also displayed.

Relative Humidity:

The values for relative humidity and dew point are displayed.

Note: While the dew point is actually graphed with the temperature, it’s a function of temperature and relative humidity. Its displayed value ended up in this area but is positioned just below the temperature graph so it’s still associated with it.

The relative humidity is graphed by itself. The color is matched to the text label. The graph scale is 0-100%; as such it doesn’t need to auto-scale, so it’s fixed.

A description (multiple sources, but arbitrary) of the relative humidity is also displayed.

Wind Direction Area:

The value for the wind direction is averaged over the last two minutes.

The wind direction description and value (albeit in small text) are displayed.

The wind direction is graphed. The color is matched to the text label. The graph scale (top to bottom) is the representation of going around a compass dial in clockwise rotation starting at North; as such it doesn’t need to auto-scale, so it’s fixed.

Precipitation Area:

The values for current precipitation accumulation, precipitation rate, number of lightning strikes, and the last lightning strike distance (if applicable) are displayed.

The precipitation accumulation and precipitation rate are graphed. The colors are matched to the text labels. Precipitation accumulation is graphed as an increasing line, while precipitation rate is graphed as a histogram. Precipitation accumulation is graphed on top on precipitation rate. The graph scale defaults to 0 to 0.5”, but does auto-scale up first to 1”, then in 1” increment as necessary.

A description (cited in the code) of the precipitation rate is also displayed.

Solar Radiation:

“Warning Uneven Road Ahead”

When I originally graphed the lux value reported from the RF receiver module I was met with a data issue. There were several discontinuities in the data (see picture “Lux data - raw.jpg”). I spent a little over a week capturing raw data early in the morning and late in the afternoon on clear sky days (no clouds to muck things up) and discovered 7 well defined discontinuities. I searched for reasons for the discontinuities before finally coming to the conclusion they were deliberate (i.e. likely sabotage by a disgruntled Engineer). I still don’t know if it’s from the actual Acurite Atlas weather station or in the RF receiver module, it really doesn’t matter. I have no access to clear data and had to come up with a way to use this corrupted data.

I eventually came up with code that detected each discontinuity and compensated for each individually. It’s not perfect, in fact I think it’s a bit messy, but it produces much cleaner data (see picture “Lux data - adjusted.jpg”). If anyone has any more information what is going on, or a better solution, I’d like to hear from you.

The lux value means little to me, so in an effort to get something more useful I converted it from lux to W/m². It’s not an ideal conversion, but it’s all I have.

The values for the current solar radiation, adjusted lux (albeit in small text), peak solar radiation, and UV index and UV risk (if applicable) are displayed.

The solar radiation is graphed. The color is matched to the text label. The maximum value I’ve witnessed to date is 1,078 W/m². The graph scale is 0 to 1,200 W/m²; as such it doesn’t need to auto-scale, so it’s fixed.

A description (cited in the code) of the lux value is also displayed.

Finished weather station display…

The finished weather station display is going to hang on my living room wall in place of the Acurite HD display; as such it has to look good.

Weather station display parts list:

Raspberry Pi Zero 2W running Raspberry Pi OS Lite (32-bit)

Pisugar S Portable 1200mAh UPS

Dell Monitor (P2418HT), 1920X1080, LED backlight, Black

VideoSecu Monitor Wall Mount, VESA 75/100 Ultra Thin Mount Bracket

USB type-A to micro USB cable

USB type-B to micro USB cable

HDMI to HDMI mini cable

Clear electronic grade silicone

Some small tie wraps

And the Acurite RF module and 6-wire cable I made to connect it to the Raspberry Pi GPIO

The monitor doesn’t have to have a touch screen; the monitor I used was left over from a previous project. It should have a USB hub in it with USB ports on both the rear and the side to make connecting a keyboard (if/when necessary) easy. The reason for the Pisugar UPS is to keep everything, except the monitor, up and running during power outages.

The battery of the Pisugar UPS and the Acurite RF module were glued to the rear of the monitor using clear electronic grade silicone (make sure to allow it to cure for 24 hours before attaching anything else). The Pisugar UPS control board is attached to the Raspberry Pi Zero 2W using the hardware that comes with it.

The Raspberry Pi Zero 2W and Pisugar control board are attached to the Pisugar battery using the Pisugar control board’s strong magnet. Everything else is accomplished using cables and tie wraps. The Pisugar UPS gets power from one of the USB ports (USB type-A) on the rear of the monitor. The Raspberry Pi’s USB port is connected to the USB hub connection (USB type-B) on the rear of the monitor. And of course the Raspberry Pi’s HDMI port is connected to the monitor’s HDMI port. All the cabling is bound up tight and held in place using tie wraps. I did have to cut off the monitor case’s cable routing plastic guides on the rear to make this all easier.

IMPORTANT NOTE: The antenna cable from the RF module must not be routed in parallel with the USB cables! It should cross the USB cables only at a right angle.

I was suffering from a lot of “bad packets” and had to move the antenna cable around to find a good location. While I was doing so I was using black electrical tape to hold the antenna in place. Once I found a good location I just left it with the electrical tape holding it in place.

See picture “Monitor rear.jpg” for rear view after everything was in place.

Finally I put in a recessed AC outlet in the wall behind where the monitor hangs to give it a clean wall appearance.

See picture “Lots of rain.jpg” for final hanging view.

weather2.c read me...

Some changes to weather.c cause me to release an updated version weather2.c.

The first change was to connect the points plotted on weather.c graphs with thick lines, making the discontinuities between the plotted points disappear. This was accomplished by adding a DrawThickLine function in weather2.c.

NOTE: I didn't apply this to the wind direction graph as the wind vane direction is "unstable" below ~4mph and the graph becomes a mess of lines.

The second change was to make wind gust more visible on the wind speed graph. In weather.c wind gusts are a yellow 5-pixel circle. In weather2.c wind gusts are a yellow 9-pixel square. So they are 40% larger/brighter and the square shape does highlight them a bit.

The third change was to how wind direction is averaged. In weather.c wind direction is averaged using a simple method that does work well for averaging just two samples, but can produce erroneous results when applied to more than two samples. In weather2.c wind direction is averaged over two minutes (four samples) using what I’ll call the ATAN2() function. This method is cited in several places on the internet.

weather3.c read me…

So I’ve been tinkering with weather2.c for a while now. I made some changes to the layout, improved wind speed and direction calculations, and a few other things.

Initialization got clean up a bit, added some initialization messages, and now pauses for 5 seconds for reading purposes before launching into the main loop.

The main loop initialization has been moved from throughout the main loop to a two stage, first pass initialization with a single flag to check rather than all the checking throughout the main loop. Moved most of the data processing from throughout the main loop to a single section between the two stages of the first pass initialization. This leaves the display area code sections to deal with just displaying.

Added a threshold variable for heat index below which heat index is ignored (float heat_index_threshold = 80;) and added a minimum delta variable between heat index and temperature below which heat index is ignored (float heat_index_temp_minimum = 2;)

Added a minimum delta variable between temperature and wind chill below which wind chill is ignored (float temp_wind_chill_minimum = 2;)

Wind speed and direction are now both averaged over the last 5 minutes.

Added a minimum threshold variable for wind speed below which wind direction is not calculated (float wind_dir_speed_threshold = 2.8;) Note: This value is dependent upon many factors (wind vane, placement, turbulence from nearby objects, etc.) and should be customize for the specific setup. Otherwise you could just set it to zero and tolerate the wind direction graph clutter at low speeds. As an aid, it also draws a dotted line for the threshold speed on the wind speed graph.

Wind gusts are now displayed as a diamond to make them more visible.

Since the precipitation rate is calculated from the precipitation accumulation spread over 6 minutes the precipitation rate calculated is actually the precipitation rate from 3 minutes in the past. As such the precipitation rate calculated is now saved as the precipitation rate 3 minutes in the past.

The solar radiation graph now auto-scales.

The general weather description has been combined into the top right corners in the: temperature, wind speed, and solar areas. It just made it easier for me to get a quick summary of the weather this way.

I’m probably going to leave it alone at this point and move on to another project. Enjoy!